# WAN Project – Quality Assurance Test Plan

### 1. Introduction

This test plan ensures the WAN infrastructure for the flagship clothing store supports business operations across all departments with reliability, security, and performance. Testing covers connectivity, redundancy, routing, security, and application availability.

### 2. Objectives

- Verify end-to-end connectivity across all departments.

- Ensure redundancy and failover between routers works correctly.

- Validate access control (only authorized traffic is allowed).

- Confirm printers, servers, and IP phones operate correctly over the WAN.

- Test application performance (file sharing, VoIP, ERP system).

- Validate scalability for future department expansion.

### 3. Scope

- In-scope:

  - Routers (ISR4331)

  - Switches (2960, 2960-24TT)

  - End devices (PCs, laptops, printers, IP phones, servers)

  - WAN routing protocols

  - VLAN & inter-VLAN routing

  - Security policies (ACLs, firewalls, authentication)

- Out-of-scope:

  - Cloud-based services not connected to WAN

  - Wireless roaming outside the site

### 4. Test Environment

- Cisco Packet Tracer (simulation) or physical lab setup.

- Devices: ISR4331 routers, 2960 switches, servers, PCs, printers, IP phones.

- WAN links configured with routing protocols (EIGRP/OSPF assumed).

- VLANs mapped per department.

### 5. Test Cases

#### 5.1 Connectivity Tests

- Ping between PCs in different departments (e.g., PC0 → PC8).

- Test file/print services (PC to printer in another department).

- Server accessibility from all departments.

#### 5.2 Redundancy & Failover

- Shut down one router (e.g., Sales & Retail) → verify traffic reroutes.

- Verify dynamic routing convergence time.

#### 5.3 Performance

- Measure latency and throughput across WAN links.

- Stress test file transfer between departments.

- Test voice call quality over IP phones.

#### 5.4 Security

- Validate ACLs (e.g., Sales PCs cannot access HR server directly).

- Test VLAN isolation.

- Verify only authorized users access shared printers/servers.

#### 5.5 Application Tests

- ERP/Inventory server accessible from Supply Chain & Retail.

- Marketing team laptops access shared file server.

- VoIP functionality between Corporate & Sales departments.

#### 5.6 User Acceptance

- Employees can log in, print, make calls, and access shared drives.

- Network behaves normally during peak business hours.

### 6. Acceptance Criteria

- 100% reachability between authorized devices.

- Failover restores connectivity within 30 seconds.

- Latency < 100 ms for inter-department traffic.

- VoIP jitter < 30 ms.

- Unauthorized access blocked.

### 7. Deliverables

- Test results log

- Issue/defect report

- Final QA sign-off report

### 8. Risks & Mitigation

- Risk: Routing loops → Mitigation: enable split horizon, route summarization.

- Risk: Single point of failure on switch → Mitigation: redundancy in future upgrade.

- Risk: Misconfigured ACLs → Mitigation: peer review before deployment.

WAN Project – Test Execution Checklist

### 1. Connectivity Testing

Goal: Ensure all authorized devices can communicate.

- Ping Test: PC0 → PC8 (Design → Corporate) → Expected: Success

- Ping Test: PC4 → Printer3 (Production → Sales) → Expected: Success

- Ping Test: Laptop2 → Server0 (Marketing → Corporate server) → Expected: Success

- Ping Test: Supply Chain PC6 → HR Server0 → Expected: Success

- Printer Access: Print test document from PC3 → Printer1 (Production) → Expected: Document prints

### 2. Redundancy & Failover

Goal: Confirm WAN can reroute if a router fails.

- Router Down Simulation: Shut down Sales & Retail router → PC0 → PC7 ping → Expected: Alternate route available

- Convergence Test: Measure time for routing tables to reconverge → Expected: <30 seconds

- Restore Router: Bring router back online → verify routes auto-update → Expected: Normal traffic resumes

### 3. Performance Testing

Goal: Check speed and responsiveness.

- File Transfer Test: Send file from PC8 (Corporate) → PC7 (Sales) → Expected: <5 sec for small files

- Latency Test: Ping between Corporate & Marketing laptops → Expected: <100ms average latency

- VoIP Test: Make call between IP Phone0 (Corporate) → PC8 softphone → Expected: Clear audio, no drops

### 4. Security Testing

Goal: Ensure only allowed access is possible.

- ACL Test 1: Try accessing HR server from Sales PC7 → Expected: Blocked

- ACL Test 2: Supply Chain PC5 access ERP server → Expected: Allowed

- VLAN Isolation Test: Attempt ping between Marketing and Supply Chain VLAN PCs → Expected: Blocked unless routed

### 5. Application Testing

Goal: Validate department-specific resources.

- ERP Server Access: Login from Supply Chain PC6 → ERP app → Expected: Success

- Marketing File Share: Laptop3 → Corporate Server shared folder → Expected: File accessible

- Printing Across WAN: Sales Laptop0 → Printer1 (Production) → Expected: Document prints

### 6. User Acceptance Testing

Goal: Simulate real-world workflows.

- Corporate Employee Workflow: PC8 → Email + File Share + Printer → Expected: All work

- Sales Workflow: Laptop0 → Access ERP + Print Invoice → Expected: Success

- Marketing Workflow: Laptop2 → Access shared drives, upload campaign material → Expected: Success

### 7. Post-Test Actions

- Document Pass/Fail for each test.

- Log any issues (with screenshots in Packet Tracer).

- Create a Defect Report if failures occur.

- Retest after fixes.